Калибровка моделей классификации данных газовых сенсоров

Calibration of gas sensor data classification models

Автор: Юлия Витюгова

Московский государственный университет имени М.В.Ломоносова

Научно-исследовательского института ядерной физики имени Д.В. Скобельцына

Лаборатория адаптивных методов обработки данных

Научные руководители: С.А. Доленко, А.О. Ефиторов

Аuthor: Iuliia Vitiugova

Lomonosov Moscow State University

Skobeltsyn Institute of Nuclear Physics

Laboratory of Adaptive Methods of Data Processing

Scientific supervisors: Sergey Dolenko, Alexander Efitorov

Аннотация

В данной научной работе рассматривается проблема хранения и обработки сенсорных данных в современной микроэлектронике и их применение в системах мониторинга. В качестве иллюстрации проведено исследование с использованием газовых датчиков. Рассматривалась эффективность методов калибровки при различных алгоритмах машинного обучения при решении задачи классификации газов. В особенности, статья выделяет важность калибровки моделей в машинном обучении для повышения эффективности предсказаний и прояснения линейности функции принадлежности.

Abstract

This scientific work focuses on the problem of storing and processing sensor data in modern microelectronics and their application in monitoring systems. An experiment was conducted using gas sensors as an illustration. The efficiency of calibration methods was examined using various machine learning algorithms to solve the problem of gas classification. In particular, the paper emphasizes the importance of model calibration in machine learning to enhance the accuracy of predictions and explain the linearity of the membership function.

Ключевые слова: сенсорные данные, газовые датчики, машинное обучение, анализ больших данные, калибровка моделей.

Keywords: sensor data, gas sensors, machine learning, big data analysis, model calibration.

Введение

В настоящее время заметны весьма значительные успехи в развитии микроэлектроники и коммуникационных технологий, повсеместное проникновение цифровых технологий привели к появлению огромных объемов сенсорных данных. Сбор, хранение и анализ сенсорных данных, являются немаловажным направлением деятельности в современном мире (I. Shafer, 2013). Сенсорные данные от датчика представляют собой временные ряды. Примерами источников данных являются системы управления производством, системы эксплуатационного мониторинга телекоммуникационных систем, системы автоматизации научных исследований. Анализ сенсорных данных позволяет контролировать изменение состояния объекта мониторинга во времени, изучать закономерности поведения, выявлять тенденции, удаленно диагностировать неисправности, принимать обоснованные решения по управлению объектом. Задача хранения сенсорных данных, таким образом, является актуальной и уже имеет множество решений. Недавний систематический поиск программного обеспечения баз данных временных рядов обнаружил 83 программных системы, и не похоже, чтобы на этом развитие темы закончилось (A. Bader, 2017). Разнообразие решений вызвано разнообразием условий задачи, в числе которых количество датчиков, темп поступления данных, длительность хранения, преобладание запросов на запись или на чтение, типы запросов на чтение, экономические ограничения. В исследуемой работе были рассмотрены данные с газовых сенсоров, представляющие собой простые временные ряды. Суть эксперимента заключалась в устранении утечек в трубопроводе с помощью создания газовых сенсоров. Проверка трубопровода основана на оценках: с помощью численных моделей, на основе параметров работы трубопровода, обнаружения акустической эмиссии и т.д. В нашем случае модель предсказывала значение одного из двух классов в ходе которых выступали газы: пропан и водород. Проводился анализ функции ответа выходов модели на валидационной выборке, приводя ее к линейному виду для того, чтобы оценить степень принадлежности примера к тому или иному классу. По итогу рассматривалась эффективность методов калибровки при различных алгоритмах машинного обучения при решении задачи бинарной классификации.

Модели машинного обучения помогают принимать решения в весьма сложных и ответственных задач, решение которых без участия человека казалось ранее недоступным. Наряду с этим одной из важнейших и до конца нерешённых теоретических и практических проблем в машинном обучении является калибровка моделей. На протяжении обучения в связи с разными причинами итоговая модель может демонстрировать разную эффективность для всевозможных примеров. Вытекающий фактор — функция числа, показывающая принадлежность примера к тому или иному классу не является линейной. Вместе с тем при выборе по порогу предполагается линейное и одинаковое поведение всех выходов сети. Калибровкой модели называют задачу приведения функции принадлежности к линейному виду.

Introduction

Currently, there are significant advances in the development of microelectronics and communication technologies, and the widespread use of digital technologies has led to the emergence of vast amounts of sensor data. The collection, storage, and analysis of sensor data are essential activities in the modern world (I. Shafer, 2013). Sensor data from sensors represent time series. Examples of data sources include production management systems, operational monitoring systems for telecommunications systems, and automation systems for scientific research. The analysis of sensor data allows for the monitoring of changes in the state of the monitored object over time, the study of behavioral patterns, the identification of trends, remote diagnostics of faults, and the making of informed decisions about object management. Therefore, the task of storing sensor data is relevant and has already been solved in many ways. A recent systematic search for time series database software found 83 software systems, and it does not seem that the development of this topic has ended (A. Bader, 2017). The diversity of solutions is due to the diversity of task conditions, including the number of sensors, the rate of data arrival, the duration of storage, the predominance of write or read requests, the types of read requests, and economic constraints. In the present study, data from gas sensors, representing simple time series, were considered. The essence of the experiment was to eliminate leaks in a pipeline using gas sensors. The pipeline check was based on estimates using numerical models, based on pipeline operating parameters, detection of acoustic emission, etc. In our case, the model predicted the value of one of two classes, in which gases were involved: propane and hydrogen. An analysis of the model output function was performed on the validation set, bringing it to a linear form, to evaluate the degree of membership of the example to one or another class. As a result, the effectiveness of calibration methods was considered for various machine learning algorithms in solving the problem of binary classification.

Machine learning models help make decisions in very complex and responsible tasks, the solution of which without human involvement seemed previously unattainable. Along with this, one of the most important and still unsolved theoretical and practical difficulties in machine learning is model calibration. During training, for various reasons, the resulting model may demonstrate different efficiency for various examples. The resulting factor is a non-linear membership function that shows the membership of an example to one or another class. However, when choosing by threshold, linear and the same behavior of all network outputs are assumed. Model calibration is the task of bringing the membership function to a linear form.

Данные

Используемые в данной работе экспериментальные данные были получены на химическом факультете МГУ кафедры неорганической химии в лаборатории химии и физики полупроводниковых и сенсорных материалов, руководитель группы ст.н.с., к.х.н. В.В.Кривецкий. В ходе эксперимента проводились измерения и запись данных с газовых датчиков для мониторинга атмосферного воздуха с целью обнаружения утечек. Использовался синтез газочувствительных материалов с помощью одностадийного метода пиролиза с распылением пламени для изготовления датчиков. В настоящем исследовании были использованы три материала: чистый SnO2, модифицированный золотом SnO2 и биметаллические Au и Pd, модифицированные SnO2.  Датчики помещали в изолированную проточную камеру из политетрафторэтилена. Поток воздуха отбирался снаружи здания химического факультета МГУ через трубку из ПТЭФ, снабженную керамическим пылевым фильтром с помощью мембранного вакуумного насоса. Наружный поток был зафиксирован с помощью дроссельной заслонки. Газовые баллоны использовались для добавления газов с помощью регуляторов расхода. Генератор чистого воздуха использовался для подачи воздуха, разбавления потока углеводородов, а также для формирования потока воздуха через камеру датчика без примеси метана или пропана. Смешивание газов продолжалось 12 ч и состояло из трех этапов по 1 ч каждый с поступлением газа в различных концентрациях как для метана, так и для пропана. Каждая стадия поступления газа отделялась от следующей потоком окружающего воздуха в течение 1 ч. Приготовление газовой смеси было полностью автоматизировано с использованием контроллеров массового расхода и заслонок, управляемых специальным программным обеспечением для ПК. Датчики работали в режиме модуляции рабочей температуры, с линейными ступенями нагрева и охлаждения и стабильными температурными интервалами 5 секунд при низкой и высокой температуре. Высокая температура была установлена равной 500°C для всех 3 датчиков, низкая температура была установлена равной 100°C. Стадия нагрева длилась 35 секунд, охлаждения – 15 секунд. Таким образом, один цикл модуляции температуры длился 60 секунд. Примененный протокол смешивания газа позволил получить 60 циклов измерений концентрации газа. Сопротивление чувствительного слоя измеряли в режиме постоянного тока смещения. Для реализации программы режима работы датчика и сбора данных использовался лабораторный электронный блок управления.

Data

The experimental data used in this study were obtained at the Department of Inorganic Chemistry of Moscow State University in the laboratory of chemistry and physics of semiconductor and sensor materials, led by a senior researcher, Ph.D. V.V. Krivetsky. During the experiment, measurements and data recording were carried out from gas sensors for monitoring atmospheric air to detect leaks. Gas-sensitive materials were synthesized using a one-stage pyrolysis method with flame spraying to make sensors. In this study, three materials were used: pure SnO2, gold-modified SnO2, and bimetallic Au and Pd, modified with SnO2. The sensors were placed in an isolated flow chamber made of polytetrafluoroethylene. The airflow was taken from outside the building of the Moscow State University chemical faculty through a polytetrafluoroethylene tube equipped with a ceramic dust filter using a membrane vacuum pump. The external flow was fixed using a throttle valve. Gas cylinders were used to add gases using flow regulators. A generator of clean air was used for supplying air, diluting the flow of hydrocarbons, and forming an air flow through the sensor chamber without methane or propane impurities. Gas mixing lasted for 12 hours and consisted of three stages of 1 hour each, with the supply of gas in various concentrations for both methane and propane. Each gas supply stage was separated from the next by the flow of ambient air for 1 hour. The preparation of the gas mixture was fully automated using mass flow controllers and valves controlled by special software for PCs. The sensors operated in the mode of modulation of the working temperature, with linear heating and cooling steps and stable temperature intervals of 5 seconds at low and high temperatures. The high temperature was set equal to 500 °C for all three sensors, and the low temperature was set equal to 100 °C. The heating stage lasted for 35 seconds, and cooling lasted for 15 seconds. Thus, one temperature modulation cycle lasted for 60 seconds. The gas mixing protocol used allowed for obtaining 60 cycles of gas concentration measurements. The resistance of the sensitive layer was measured in the constant bias current mode. A laboratory electronic control unit was used to implement the sensor operation and data collection program.

Оценка прогнозирования

При калибровке модели мы пытаемся масштабировать предсказания модели так, чтобы распределение и поведение оценки вероятности для каждого класса было аналогично распределению и поведению вероятности для каждого класса, наблюдаемым в обучающих данных.

Рассмотрим задачу классификации с  k – количеством классов, входными данными x∈Х и меткой y∈Y=1,…, k. Пусть h - нейронная сеть и при ее действии на X, получаем: hX=Y,P, где Y – предсказание класса, а P – вероятность достоверности предсказания, подлежащая калибровке. Введем понятие идеальной калибровки, которая определяется как:

PY=YP= P=p,  ∀p∈0,1 При идеальной калибровке значения оценки принадлежности должны соответствовать доле правильно классифицированных значений. Например, при рассмотрении 100 примеров, каждый с вероятностью правильного ответа 0.8, ожидая, что 80 из них должны быть правильно классифицированы. В реальных условиях достичь идеальной калибровки практически невозможно, но качество калибровки возможно оценить с помощью некоторых метрик:

Prognosis evaluation

When calibrating a model, we try to scale the model's predictions so that the distribution and behavior of the probability estimate for each class are similar to the distribution and behavior of the probability for each class observed in the training data.

Let's consider a classification problem with k classes, input data x∈Х, and label меткой y∈Y=1,…, k. Let h be a neural network, and when it operates on X, we obtain hX=Y,P,;, where Yis the predicted class, and  P is the probability of the prediction's reliability, subject to calibration. Let's introduce the concept of perfect calibration, which is defined as follows:

PY=YP= P=p,  ∀p∈0,1

In the case of perfect calibration, the values of the membership estimate should correspond to the proportion of correctly classified values. For example, considering 100 examples, each with a probability of a correct answer of 0.8, we expect that 80 of them should be correctly classified. In real conditions, achieving perfect calibration is practically impossible. The quality of the model's calibration can be evaluated using the following metrics:

Диаграмма калибровки

Диаграмма калибровки является распространенным средством для иллюстрации свойств вероятностных систем прогнозирования. Для построения диаграммы калибровки требуется оценить точность и уверенность модели по конечным выборкам, затем оба значения визуализируются в виде гистограммы с идентификационной линией (Рис.5). Если модель идеально откалибрована, то диаграмма должна построить функцию идентичную идеальной калибровки. В случае, если идентификационная линия ниже диагонали, тогда предсказанные оценки модели слишком велики, если ниже — слишком малы.  Для удобства дальнейшего изложения введем понятие биннинга. Значения принадлежности объекта к классу модели группируются по оценки вероятности в интервал ячеек  M и рассчитывается точность каждой ячейки. Данный алгоритм называется биннингом. Далее создается множество индексов выборок Bm, значения предсказаний, которые попадают в интервал    Im =m-1М, mМ. Рассчитать уверенность и точность модели можно по следующим формулам:

accBm= 1Bmi∈Bm1yi= yi ,

confBm=1Bmi∈Bmpi

Calibration plot

The calibration plot is a common tool for illustrating the properties of probabilistic forecasting systems. To construct a calibration plot, it is necessary to estimate the accuracy and confidence of the model on finite samples, and then both values are visualized as a histogram with an identification line. If the model is perfectly calibrated, the diagram should plot a function identical to perfect calibration. If the identification line is below the diagonal, then the model's predicted scores are too high, if it is below, they are too low. For convenience, let's introduce the concept of binning: the values of an object's membership in the model class are grouped by probability estimates into the interval of cells M, and the accuracy of each cell is calculated. This algorithm is called binning. Then a set of sample indexes 𝐵! is created, which are the values of the predictions that fall within the interval  Im =m-1М, mМ. The confidence and accuracy of the model can be calculated using the following formulas:

accBm= 1Bmi∈Bm1yi= yi ,

confBm=1Bmi∈Bmpi

Методы калибровки

Калибровка функции принадлежности — это операция масштабирования, которая применяется после того, как предсказания были сделаны моделью. Рассмотрим выходы нейронной сети: модель, получая входные данные xi и метки yi, получает прогнозируемые метки yi для класса k и выводит оценки принадлежности pi объектов к классам, а также логит-функции  zi∈R.

Биннинг гистограмм (Zadrozny, 2001) — это непараметрический метод калибровки. Вслед за биннингом некалиброванных предсказаний pi, следует задача оптимизации по параметрам θ. Каждой ячейке  B1,…,BM присваивается калиброванный параметр θ, таким образом, чтобы минимизировать функцию потерь:

minθ1,…θmm=1Mi=1n1am≤ pi< am+1θi-yi2

Bm∈am,am+1]

где  1 - функция индикатора. Учитывая фиксированные границы ячеек, решение приводит к θm, которые соответствуют среднему числу образцов положительного класса в ячейке Bm.

Изотонная регрессия (Zadrozny, 2001) — это наиболее распространенный непараметрический метод калибровки, изучающий кусочно-постоянную функцию f для преобразования некалиброванных выходов:  q = f p . Решение задачи оптимизации идентично биннингу гистограмм, но существует дополнительное условие:

minMθ1,…θMα1,…,αM+1m=1Mi=1n1am≤ pi< am+1θi-yi2

0= a1 ≤ a2 ≤…≤ aM+1=1,

θ1≤θ2≤θM

Где M — число интервалов ячеек, a1,….,aM+1 — границы интервалов. При такой параметризации изотонная регрессия является строгим обобщением биннинга гистограмм, в котором границы ячеек и предсказания ячеек совместно оптимизируются.

Байесовский Бинниг (Naeini, 2015) является ансамблевым методом. Он осуществляет комбинацию нескольких биннингов с разным числом ячеек, создавая схему биннинга s, которая представляет собой пару (M,L) из числа ячеек M и соответствующего разбиения L. Если биннинг гистограмм и изотоническая регрессия дают единую схему биннинга, то Байесовский биннниг рассматривает пространство S всех возможных схем биннинга для набора данных валидации D. Оценка принадлежности выводится с помощью линейной комбинации оценок биннингов с весами, применяя байесовское усреднение вероятностей:

Pqtepte,D=s∈SPqte,  S=spte,D

= s∈SPqtepte, S=s, DPS=s D#

где   Pqtepte, S=s, D  — калиброванная вероятность с использованием схемы     биннинга S.

Температурное шкалирование придумано специально для калибровки моделей нейронных сетей. Метод температурного шкалирования считается самым простым в реализации, при этом дает весьма хорошие результаты.Идея метода заключается в том, что он работает непосредственно на логит-функциях zi, а не на оценках принадлежности pi. Логит-фукнции масштабируются с помощью скалярного параметра T>0 для всех классов. Оптимальный параметр T получается из минимизации отрицательного логарифмического правдоподобия для отложенного набора данных.                 T – называют «температурой», и она как бы «смягчает» функцию softmax (т. е. повышая выходную энтропию), T не зависит от результата классификации, поэтому точность калибруемого алгоритма не меняется. Калиброванное значение принадлежности может быть получено путем:

qi'=maxkσSMziTk

Calibration methods

Calibration of the membership function is a scaling operation that is applied after the model has made predictions. Let's consider the outputs of a neural network: the model, receiving input data xi  and labels yi, receives predicted labels yi for class k and outputs estimates of object membership in classes pi, as well as logit functions zi∈R.

Histogram binning (Zadrozny, 2001) is a non-parametric calibration method. Following the binning of uncalibrated predictions pi, an optimization problem is solved for the parameters θ. Each cell B1,…,BM is assigned a calibrated parameter θ, to minimize the loss function:

minθ1,…θmm=1Mi=1n1am≤ pi< am+1θi-yi2

Bm∈am,am+1]

where 1 is an indicator function. Given the fixed cell boundaries, the solution leads to к θm, which corresponds to the mean number of positive class samples in cell Bm.

Isotonic regression (Zadrozny, 2001) is the most common non-parametric calibration method, studying the piecewise constant function f for transforming uncalibrated outputs:  q = f p . The optimization solution is identical to histogram binning, but there is an additional condition:

minMθ1,…θMα1,…,αM+1m=1Mi=1n1am≤ pi< am+1θi-yi2

0= a1 ≤ a2 ≤…≤ aM+1=1,

θ1≤θ2≤θMWhere M is the number of cell intervals, a1,….,aM+1  are the interval boundaries. With this parameterization, isotonic regression is a strict generalization of histogram binning, in which cell boundaries and predictions are jointly optimized.

Bayesian Binning (Naeini, 2015) is an ensemble method. It combines several binnings with different numbers of cells, creating a binning scheme s, which is a pair (M,L) of the number of cells M and the corresponding partition L. If the histogram binning and isotonic regression give a unified binning scheme, then Bayesian binning considers the space S of all possible binning schemes for the validation dataset D. The membership estimate is derived using a linear combination of binning estimates with weights, applying Bayesian averaging of probabilities:

Pqtepte,D=s∈SPqte,  S=spte,D

= s∈SPqtepte, S=s, DPS=s D#

where Pqtepte, S=s, D  is the calibrated probability using the binning scheme S.

Temperature scaling was invented specifically for calibrating neural network models. The temperature scaling method is considered the simplest to implement, while providing very good results. The idea of the method is that it works directly on the logit functions zi, rather than on estimates of membership pi. Logit functions are scaled using a scalar parameter  𝑇 > 0 for all classes. The optimal parameter 𝑇 is obtained by minimizing the negative log-likelihood for a held-out dataset. 𝑇 is called the "softens" softmax function (i.e., increasing output entropy), 𝑇 does not depend on the classification result, so the accuracy of the calibrated algorithm does not change.

The calibrated membership value can be obtained by:

qi'=maxkσSMziTk

Результаты

В данном вычислительном эксперименте были рассмотрены выходы четырех алгоритмов машинного обучения: Многослойного персептрона (МП), Метода опорных векторов (МОВ), Стохастического градиентного спуска (СГС) и Случайного леса (СЛ). Используемые данные были разделены на обучающий и тестовый набор, которые содержали 5292, 2808 примеров. В качестве базовых параметров обучения модели нейронной сети были взяты следующие: персептрон c логистическая сигмоидальной передаточной функцией для скрытых слоев, для выходного слоя использовалась линейная. Далее к моделям были применены методы калибровки: Биннинг гистограмм, Изотонная регрессия, Байесовский бинниг и Температурное шкалирование. Рассчитаны оценочные значения эффективности (площадь под графиком ROC — AUC) моделей до и после применения методов калибровки.

Results

In this computational experiment, the outputs of four machine learning algorithms were considered: Multilayer Perceptron (MP), Support Vector Machine (MOV), Stochastic Gradient Descent (SGD) , and Random Forest (RF). The following were taken as the basic training parameters of the neural network model: a perceptron with a logistic sigmoid transfer function for hidden layers and linear for the output layer. Next, the models were subjected to calibration methods: Binning Histograms, Isotonic Regression, Bayesian Binning, and Temperature Scaling. Estimated effectiveness values (area under the ROC curve - AUC) of the models were calculated before and after applying calibration methods. Reliability diagrams and histograms of model confidence were obtained.

Заключение

При нейросетевом решении задачи бинарной классификации, полученные метки и оценки принадлежности примеров к классам прошли постобработку с помощью методов калибровки: Гистограммного биннинга, Изотонной регрессии, Байесовского биннинга и Температурного шкалирования. По результатам выполнения данной работы можно сделать следующие выводы:

- Для модели МП классификатора лучшие оценки эффективности показали методы Температурного шкалирования и Байесовского биннинга, наблюдалось ухудшение качества при методах Изотонной регрессии и Гистограммного биннинга.

- Для МОВ модели высокие значения получены при применении метода Температурного шкалирования, другие методы показали менее высокие результаты.

- Для классификатора СГС самая высокая оценка эффективности была при применении метода Изотонной регрессии.

- При применении метода Изотонной регрессии, получены наилучшие результаты для модели СЛ.

Conclusion

In the case of a neural network solution to the problem of binary classification, the labels and membership estimates of examples to classes underwent post-processing using calibration methods: Histogram Binning, Isotonic Regression, Bayesian Binning, and Temperature Scaling. Based on the results of this course work, the following conclusions can be illustrated.

- For the MP classifier model, the best effectiveness estimates were shown by the Temperature Scaling and Bayesian Binning methods, with deterioration of quality observed for the Isotonic Regression and Histogram Binning methods.

- For the MOV model, high values were obtained when applying the Temperature Scaling method, while other methods showed lower results.

- For the SGD classifier, the highest effectiveness estimate was obtained when applying the Isotonic Regression method.

- When applying the Isotonic Regression method, the best results were obtained for the RF model.

Список литературы

- I. Shafer, R.R. Sambasivan, A. Rowe, G.R. Ganger, Specialized storage for big numeric time series, 2013

- A. Bader, O. Kopp, M. Falkenthal. “Survey and comparison of open source time series databases, 2017.

- Chuan Guo, Geoff Pleiss, Yu Sun, Kilian Q. Weinberger. On Calibration of Modern Neural Networks, 2017.

- Max Kuhn. Applied Predictive Modeling, 2017.

- Valeriy V. Krivetskiy, Matvei D. Andreev, Aleksandr O. Efitorov, Alexander M. Gaskov. Statistical shape analysis of temperature modulated metal oxide gas sensor response for improved selectivity of hydrocarbons detection in real atmospheric conditions, 2020.

- Jochen Bröcker and Leonard A. Smith. Increasing the Reliability of Reliability Diagrams, 2007.

- Alexandru Niculescu-Mizil, Rich Caruana. Predicting Good Probabilities with Supervised Learning, 2005.

- Bianca Zadrozny Charles Elkan. Obtaining calibrated probability estimates from decision trees and naive Bayesian classifiers, 2001.

- Tim Leathart, Maksymilian Polaczuk, Temporal Probability Calibration, 2020.

- Taejong Joo, Uijung Chung, Min-Gwan Seo, Being Bayesian about Categorical Probability, 2020.

- Tim Leathart Eibe Frank Bernhard Pfahringer Geoffrey Holmes, On Calibration of Nested Dichotomies, 2018.

References

- I. Shafer, R.R. Sambasivan, A. Rowe, G.R. Ganger, Specialized storage for big numeric time series, 2013

- A. Bader, O. Kopp, M. Falkenthal. “Survey and comparison of open-source time series databases, 2017.

- Chuan Guo, Geoff Pleiss, Yu Sun, Kilian Q. Weinberger. On Calibration of Modern Neural Networks, 2017.

- Max Kuhn. Applied Predictive Modeling, 2017.

- Valeriy V. Krivetskiy, Matvei D. Andreev, Aleksandr O. Efitorov, Alexander M. Gaskov. Statistical shape analysis of temperature modulated metal oxide gas sensor response for improved selectivity of hydrocarbons detection in real atmospheric conditions, 2020.

- Jochen Brö cker and Leonard A. Smith. Increasing the Reliability of Reliability Diagrams, 2007.

- Alexandru Niculescu-Mizil, Rich Caruana. Predicting Good Probabilities with Supervised Learning, 2005.

- Bianca Zadrozny Charles Elkan. Obtaining calibrated probability estimates from decision trees and naive Bayesian classifiers, 2001.

- Tim Leathart, Maksymilian Polaczuk, Temporal Probability Calibration, 2020.

- Taejong Joo, Uijung Chung, Min-Gwan Seo, Being Bayesian about Categorical Probability, 2020.

- Tim Leathart Eibe Frank Bernhard Pfahringer Geoffrey Holmes, On Calibration of Nested Dichotomies, 2018.

Приложение

Supplementary materials

Calibration plots